Optimizing Leak Management Strategies for Water Networks: 
A Cost-Minimization Model for Logan, Utah

Ty Gilbert

Dr. David E. Rosenberg

CEE 6410

December 11, 2025

Abstract

This project develops an optimization framework for leak management in municipal water distribution networks, specifically examining the high-elevation subsystem of Logan, Utah. This subsystem, supplied by Dewitt Springs, presents a unique challenge: it serves critical institutional consumers (Utah State University) via a mix of aging transmission mains and residential distribution pipes. The study formulates a mixed-integer linear programming (MILP) model to minimize the cumulative costs of acoustic inspection, infrastructure repair, and treated water loss over a one-year planning horizon. By isolating this subsystem, the model evaluates the specific economic trade-offs between repairing high-consequence large-diameter transmission mains versus high-frequency small-diameter residential leaks. To ensure operational feasibility, the optimization incorporates budget limits and volumetric delivery constraints that maintain service capacity for the Castle Hills and Upper pressure zones. The output provides utility managers with a prioritized intervention schedule that maximizes hydraulic reliability while minimizing financial loss.

Introduction

Municipal water utilities worldwide face a compounding crisis: aging infrastructure is increasing the rate of non-revenue water, while maintenance budgets remain stagnant or shrink. This financial strain is exacerbated by growing water scarcity, which raises the economic and environmental cost of every lost gallon. The resulting budget gap forces utilities into a reactive trap, where funds are consumed by emergency repairs, leaving little for preventive care. The fundamental challenge for decision-makers is not simply how to fix leaks, but when and where to intervene to maximize the value of every dollar spent. This creates a complex economic optimization problem where the active costs of inspection and repair must be continuously weighed against the passive, yet cumulative, financial loss of treated water escaping the system.

While previous research has applied stochastic optimization to large-scale municipal planning (e.g., Rosenberg & Lund, 2009) or focused on the efficacy of specific detection technologies (Nagapurkar et al., 2025), there is a need for models that address the specific operational trade-offs within mixed-asset subsystems. Specifically, managers must decide whether to prioritize the inspection of large-diameter transmission mains—which fail less frequently but with catastrophic volume loss—or small-diameter distribution pipes, which suffer from high-frequency, lower-volume background leakage.

This project addresses this gap by developing a MILP model applied to the Dewitt Springs subsystem in Logan, Utah. This subsystem serves as an ideal case study because it contains both critical transmission infrastructure and a dense residential distribution network. By modeling this specific hydraulic loop, the project aims to move beyond reactive "break-and-fix" policies to a proactive, mathematically optimized schedule. The primary contribution of this work is a decision support tool that explicitly quantifies the value of intervention, allowing utility managers to justify maintenance schedules based on minimized total system cost rather than simple age or failure history.

Literature Review

Effective municipal water management requires balancing intervention costs against the passive losses of aging infrastructure. Recent research highlights three critical components for optimizing this balance: detection efficacy, network optimization, and risk valuation.

Comparative Efficacy of Detection Methods. Selecting the appropriate detection technology is the first step in cost minimization. Nagapurkar et al. (2025) conducted a comparative analysis of acoustic detection versus AI-based satellite detection. While satellite methods offer speed over large surface areas, the study determined that acoustic detection currently provides superior economic returns for municipal systems, yielding 50% higher net benefits due to better leak isolation capabilities. Based on these findings, this project utilizes acoustic monitoring as the primary detection variable.

Optimization under Uncertainty. The scheduling of inspections and repairs must address system uncertainties. Kapelan et al. (2005) introduced stochastic approaches to least-cost design, demonstrating that ignoring uncertainty in future water demand leads to inadequate system scaling. Building on this, Sirsant and Reddy (2022) expanded network optimization to include life-cycle costs (LCC). These studies establish that effective models must minimize costs over a time horizon rather than focusing solely on immediate capital expenditures.

Risk Assessment and Infrastructure Failure. Accurate optimization requires precise definitions of failure probability. Barfuss (2023) establishes critical baselines by quantifying aggregate failure rates, noting that pipes with diameters smaller than 12 inches fail at a rate five times higher than larger transmission mains. This is particularly relevant to the Logan case study, which contains both large diameter transmission lines (Dewitt Canyon line) and small diameter residential lines (Castle Hills). Complementing this, Cen et al. (2023) utilized Random Forest algorithms to predict risk based on pipe material and age. This project synthesizes these approaches by applying linear programming to the specific hydraulic topology of the Logan, Utah high-elevation zones.

Problem Formulation

Dimensions:

- i : Set of pipe segments in the subsystem (Castle Hills, Hillcrest, Canyon Transmission).

- t : Set of time periods (4 Quarters, representing one fiscal year).

- z : Set of pressure zones {CastleHills, Upper, USU}.

Parameters:

- Lratei,t : Expected water loss rate from an unfixed leak in pipe i at time t [m³/day].

- Cinspecti,t : the cost of inspecting pipe i at time t using acoustic methods [$].

- Crepairi,t : Estimated cost to repair a leak in pipe i at time t [$].

- Cwatert : Cost of lost water in time t [$/m³].

- Bt : Total available budget in period t [$].

- Dz,t : Mandatory water demand for zone z in period t (e.g., USU demand) [m³].

Decision variables:

- Xinspecti,m,t : The decisions of whether to inspect pipe i with method m at time t. Binary variable: 1 = pipe i is inspected with method m in time t, and 0 = otherwise.

- Yrepairi,t : The decisions of whether to repair pipe i at time t. Binary variable: 1 = repair pipe i at time t, 0 = otherwise.

Other Variables

- Wlosti,t : Volume of water lost from pipe i at time t [m³].

- Supplytotalt : Available water supply per time period t from Dewitt Springs [m³].

Objective Function:

- Minimize the total cost of inspections, repairs, and water loss over all pipes and time periods:

MinZ=i,t​Cinspecti,t⋅Xinspecti,t+Crepairi,t⋅Yrepairi,t+Cwatert⋅Wlosti,t

Constraints

- Total spending on inspection and repairs must be less than or equal to the budget for each time period t.

m​Cinspecti,t⋅Xinspecti,t+Crepairi,t⋅Yrepairi,t≤Bt, ∀t

- A pipe cannot be repaired unless it is inspected in the same period.

Yrepairi,t≤Xinspecti,t,∀i,t

- The system must deliver sufficient water to meet the demand of the zones (specifically USU and residential areas), accounting for losses. This constraint represents the "Node receiving enough water" check.

Supplytotal-iWlosti,t≥zDz,t, ∀t

- This constraint calculates water loss based on the repair history. If a repair has occurred in any period up to the current time t, the water loss becomes zero.

Wlost(i,t)=Lrate(i)⋅1-τ=1tYi,τ, ∀i,t

- To prevent redundant spending, a specific pipe segment can be repaired at most once during the planning horizon.

tY(i,t)≤1, ∀i

Steps to Solve

The execution of this project follows the following workflow:

- Data Aggregation: Pipe characteristics (diameter, material, age) and topological data are extracted from the Logan City Drinking Water System Master Plan (2016) specifically for the Castle Hills and Upper pressure zones.

- Parameter Estimation:

  - Leak rates (Lrate) are estimated using Barfuss (2023) failure rate tables dependent on pipe diameter.

  - Costs (Cinspect,Crepair) are derived from municipal standard rates.

- Model Implementation: The MILP formulation is coded into GAMS.

- Baseline Scenario Execution: The model is run under current budget conditions to determine the optimal inspection schedule.

- Sensitivity Analysis: The budget parameter (Bt) is varied to observe how the optimal strategy shifts between repairing large transmission mains (rare but high volume) versus small distribution pipes (frequent but lower volume).

Results

The MILP model was executed in GAMS to minimize total operational costs for the Dewitt Springs subsystem over a four-quarter planning horizon. The optimization successfully converged to an optimal solution with a total cost of $238,620.

Optimal Intervention Schedule

The model identified a prioritized schedule for inspections (X) and repairs (Y) that adhered to the quarterly budget constraints while satisfying system demand.

- Quarter 1 (Q1): The model prioritized the repair of one residential pipe (Dist_CastleH_01) in the Castle Hills zone. This suggests that tackling low-cost, high-frequency leaks early provided immediate savings.

- Quarter 2 (Q2): The model aggressively targeted the high-consequence infrastructure, repairing the first major transmission main (Main_Dewitt_01). It also repaired another residential segment (Dist_CastleH_02). This utilized the larger budget available in Q2 ($45,000).

- Quarter 3 (Q3): The model repaired the second transmission main (Main_Dewitt_02) and a third residential segment (Dist_CastleH_03).

- Quarter 4 (Q4): No repairs were scheduled, likely because the remaining budget was insufficient or the marginal cost of repair exceeded the water loss savings for the remaining pipes (Dist_Upper_01 and Dist_Upper_02).

Cost Analysis Breakdown

The total objective function value of $238,620 was distributed across three cost categories:

- Inspection Costs: $3,900 (1.6%). This low percentage indicates that inspection is a high-return activity; the cost of finding a leak is negligible compared to the value of fixing it.

- Repair Costs: $74,000 (31.0%). This represents the direct capital investment in infrastructure improvement.

- Water Loss Costs: $160,720 (67.4%). Despite the interventions, the majority of the total system cost still stems from the value of lost treated water. This highlights the severe economic penalty of "non-revenue water" and suggests that increasing the maintenance budget could yield significant long-term savings by allowing for earlier or more extensive repairs.

System Reliability

The model confirmed that the volumetric delivery constraints were met in all quarters. However, two pipes (Dist_Upper_01 and Dist_Upper_02) were left unrepaired for the entire year, resulting in continuous water loss (Wlost>0 for Q1-Q4). This "run-to-failure" decision for these specific segments implies that under the strict budget constraints provided, it was more economical (or necessary) to tolerate these specific leaks than to displace the budget for the larger transmission repairs.

Conclusion

This project provides a rigorous framework for decision-making in the Logan water distribution system. By defining the trade-offs between inspection costs and water loss, the model moves beyond reactive maintenance to a proactive, value-based approach. The application to the Dewitt Springs subsystem demonstrates that targeted interventions, prioritized by pipe diameter and failure risk, can significantly reduce operational waste while maintaining the high reliability required by critical users like Utah State University.

References

Barfuss, S. L. (2023). Water Main Break Rates in the USA and Canada: A Comprehensive Study. Utah State University, Utah Water Research Laboratory. https://digitalcommons.usu.edu/water_rep/682/

Cen, H., Huang, D., Liu, Q., Zong, Z., & Tang, A. (2023). Application Research on Risk Assessment of Municipal Pipeline Network Based on Random Forest Machine Learning Algorithm. Water, 15(10), 1964. https://doi.org/10.3390/w15101964

Gilbert, T. (2025). CEE-6410-Gilbert: Leak Management Semester Project. GitHub. https://github.com/ty-gilbert/CEE-6410-Gilbert/tree/main/Leak%20Management%20Semester%20Project

Google. (2025). Response to prompt regarding constraints for water pipe inspection and repair model. [Large language model]. https://gemini.google.com/share/4af0045aa198

Hansen, Allen & Luce, Inc. (2016). Logan City Drinking Water System Master Plan: Final Report.

Kapelan, Z., Babayan, A. V., Savic, D. A., Walters, G. A., & Khu, S. T. (2004). Two new approaches for the stochastic least cost design of water distribution systems. Water Science & Technology: Water Supply, 4(5-6), 355-363. https://doi.org/10.2166/ws.2004.0126

Nagapurkar, P., Sharma, N., Garcia, S., & Nimbalkar, S. (2025). Evaluating Acoustic vs. AI-Based Satellite Leak Detection in Aging US Water Infrastructure: A Cost and Energy Savings Analysis. Smart Cities, 8(4), 122. https://doi.org/10.3390/smartcities8040122

Sirsant, S., & Reddy, M. J. (2022). Performance-based multi-objective design and expansion of water distribution networks considering life cycle costs and future demands. Water Science & Technology: Water Supply, 22(2), 1388-1408. https://doi.org/10.2166/ws.2021.344

Zhang, S., & Wang, Y. (2024). Research on Water Price and Quantity to Meet the Basic Living Needs of Urban Residents Based on Water Conservation. Water Resources Management, online first. https://doi.org/10.1007/s11269-024-03750-x

Appendix

Figure 1. Water Distribution Flowchart.